| ##### RISK ASSESSMENT | ### Use of Bouncy Castles/Inflatables at Outside Events 26/27 |  |  |
| --- | --- | --- | --- |
| Establishment: | Assessment by: Justin Leader/Katie Ball | Date: |  |
| Next Review Date Due: 15th April 2027 | Manager Approval: Katie Ball | Date: |  |

| ## Hazard/Risk | Who is at Risk? | How can the hazards cause harm? | Normal Control Measures | Are Normal Control Measures Y/N/NA |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | In Place | Adequate |
| Use of external companies/ contractors | All Persons | Accidents/property damage might not cover cost of repairs and personal injury claims. | - Operator conforms to PIPA/ADIPS/RPii or equivalent scheme. - Risk assessments provided. - Appropriate public liability insurance in place (£10m indemnity). | Y | Y |
| Using poorly maintained equipment | All Persons | Electric shock/burns Cuts/abrasions, broken limbs, muscular skeletal and other physical injuries | - Thoroughly examined every 12 months by a registered inspection body. The blower, inflatable and the electrical system should be examined and a certificate issued. These should be checked before use and the pressure of the inflatable checked. - Inspected prior to the first use on any day following supplier/manufacturer’s instructions - Inflatable has Inspection Certification covered by BSEN 14960:2019. | Y | Y |
| Use of specialist equipment Overloading equipment Weather conditions Instability/collapse of bouncy castle Inadequate supervision Inappropriate location of equipment | All Persons | Slips/falls Cuts/abrasions, broken limbs, muscular skeletal, suffocation and other physical injuries | - Supplied/erected by competent persons. - Supervision to prevent unauthorised use and control access and egress. - Limit on numbers/age/height etc using equipment (equipment clearly marked as to its limitations of use). - Sited in an appropriate location, well away from obstacles such as fences, sufficient room on site for inflatable and blower (blower is at least 1.2 metres from the inflatable). - Attendants must constantly watch the activities on the equipment. - Attendants aware of basic rules for use eg all hard/sharp/dangerous objects such as footwear, buckles etc are removed; no bouncing on the front apron of the castle; no climbing or hanging on the outside walls etc. - Equipment must not be used in high winds. High winds are 24mph or over as stated in the current available standard BSEN 14960:2019. Wind measurements to be checked and recorded hourly. All inflatables to be secured with 380mm pegs/suitable anchor points. - In high temperatures the inflatables will be regularly checked for heat conditions of the pvc. - No smoking is permitted in high temperatures outside on the grass area. - All anchorage points must be used at all times. It must not be possible for the equipment to move around. This is for outside bookings. - Generators to be placed in a safe area with fire extinguishers. Generators and petrol blowers refilled according to the method statement. - Blowers are IP65 rated for use in all weathers. - Leads fitted with BS4343 connectors. | Y | Y |
| Safeguarding of students/children DBS checks EVO Safeguarding policy. | All Persons |  | - Enhanced DBS certificates are held by the directors of the company. - EVO has a safeguarding policy in which we adhere to. |  |  |
| Manual Handling Weighty, bulky and unwieldy load Difficulties in getting a grip of the load Individuals with pre-existing conditions Physical Injury – difficult posture Muscular skeletal disorders Sprains/strains | All Persons | Slips/falls Cuts/abrasions, muscular skeletal and other physical injuries | - Sufficient number of people to move inflatable. - Minimise carrying distance. - Individuals aware of correct lifting techniques. | Y | Y |

| Additional Control Measures (to take account of local/individual circumstances including changes such as working practices, equipment, staffing levels). | Action by Whom (list the name of the person/people who have been designated to conduct actions) | Action by When (set timescales for the completion of the actions – remember to prioritise them) | Action Completed (record the actual date of completion for each action listed) | Residual Risk Rating |
| --- | --- | --- | --- | --- |
| Site Inspection To be carried out before installation of equipment | Justin Leader Katie Ball Matt Knight Calum Leader Mark Garnham Ben Leader Lisa Tucker Ella White | Prior to installation |  |  |
| DATE OF REVIEW: 1st April 2025 | COMMENTS: Completed control measures Residual risk rating completed after installation and as risk assessment is signed off |  |  |  |
| DATE OF REVIEW: 1st April 2026 | COMMENTS: Completed control measures Residual risk rating completed after installation and as risk assessment is signed off |  |  |  |
| DATE OF NEXT REVIEW: 1st April 2027 | COMMENTS: |  |  |  |

| # RESIDUAL RISK RATING | ACTION REQUIRED |
| --- | --- |
| VERY HIGH (VH) Strong likelihood of fatality/serious injury occurring | The activity must not take place at all. You must identify further controls to reduce the risk rating. |
| HIGH (H) Possibility of fatality/serious injury occurring | You must identify further controls to reduce the risk rating. Seek further advice. |
| MEDIUM (M) Possibility of significant injury or over 3 day absence occurring | If it is not possible to lower risk further, you will need to consider the risk against the benefit. Monitor risk assessments at this rating more regularly and closely. |
| LOW (L) Possibility of minor injury only | No further action required. |